RISC-V VeeR EL2 Based Hardware Packet

Buffering and Data Integrity SoC

1. INTRODUCTION:

1.1 Project Overview

The proposed project is a RISC-V VeeR EL2 Based Hardware Packet Buffering and Data Integrity SoC. The SoC is designed around the faculty-provided VeeR EL2 RISC-V processor and integrates instruction memory, data memory, a system-bus interconnect, UART, Timer, GPIO, and three application-specific hardware IPs: Packet FIFO, CRC Checker, and Packet Counter.

The main purpose of the SoC is to demonstrate how dedicated RTL hardware can be used to receive, temporarily buffer, verify, and monitor digital packet data while the RISC-V processor performs system control, configuration, decision-making, and result reporting. The architecture follows the same general SoC approach used in the reference documents, where the RISC-V processor communicates with memory-mapped hardware IPs through an AXI4-based system-bus interconnect and software running on the processor controls and reads the hardware.

The packet-processing system uses a hardware data path in which packet data generated by an RTL testbench is first stored in the Packet FIFO. The buffered packet is then provided to the CRC Checker, which recalculates the CRC and determines whether the received packet is valid or corrupted. The Packet Counter records packet-processing events, CRC errors, and FIFO-related errors. The RISC-V processor accesses the control and status registers of these IPs through the memory-mapped system bus and reports the final results through UART.

The project is intended to demonstrate a complete hardware/software integrated SoC rather than three independent IP blocks. The processor provides programmable control and monitoring, while the dedicated hardware blocks perform packet buffering, data-integrity verification, and event counting. This separation keeps repetitive packet-processing operations in hardware while allowing the processor to handle higher-level application logic.

1.2 Problem Statement

Modern embedded and SoC-based communication systems frequently receive digital data continuously or in bursts. The incoming data may arrive while the processor is occupied with other tasks, creating a requirement for temporary hardware buffering. Without sufficient buffering, packets may be lost when the rate of incoming data is greater than the rate at which the processor can service the received data.

In addition, digital data can become corrupted during transmission or processing because of noise, interference, timing problems, or hardware faults. A receiving system therefore requires an efficient mechanism to determine whether the received packet has maintained its data integrity. Performing packet buffering, integrity checking, and monitoring entirely through software can unnecessarily increase processor workload.

The proposed project addresses these problems by integrating dedicated hardware IPs into a RISC-V based SoC. The Packet FIFO provides temporary storage for incoming packets, the CRC Checker performs hardware-based integrity verification, and the Packet Counter maintains packet and error statistics. The VeeR EL2 processor controls and monitors these hardware blocks through AXI4-based memory-mapped system bus and reports the processing results through UART.

1.3 Problems Addressed by the Proposed SoC

The proposed architecture addresses the following problems:

1. Temporary packet storage: Incoming packets may arrive while the processor is busy. The Packet FIFO provides temporary storage and prevents immediate data loss due to processor servicing delays.

2. Packet integrity verification: Received packet data may contain errors. The CRC Checker recalculates the CRC and compares it with the received CRC to detect corruption.

3. Packet and error monitoring: The system needs to know how many packets were received, processed, or rejected. The Packet Counter maintains these statistics in hardware.

4. Processor workload reduction: Repetitive packet buffering and CRC calculations are performed by dedicated hardware instead of requiring the RISC-V processor to perform every low-level operation in software.

5. Hardware/software integration: The processor requires a controlled mechanism to configure the IPs and read their results. Memory-mapped registers provide this interface.

6. System-level reporting: The processed results must be observable during verification. UART provides a simple serial reporting interface for displaying packet and error information.

1.4 Proposed Solution

The proposed SoC uses the following processing sequence:

Packet Generation → Packet Buffering → CRC Verification → Packet Classification → Statistics Collection → RISC-V Processing → UART Reporting

VeeR EL2 → AXI4 Interconnect → Memory/Peripheral/IP Registers

The RTL testbench acts as the external packet source during simulation. It generates a packet containing the packet data and its corresponding CRC value. The complete packet is supplied to the SoC through the packet input interface.

The Packet FIFO receives and temporarily stores the incoming packet. FIFO status signals such as empty, full, overflow, and underflow are used to monitor the buffering operation.

The CRC Checker receives the buffered packet and independently calculates the CRC from the received packet data using the configured CRC algorithm. The calculated CRC is compared with the CRC associated with the received packet. A matching result indicates that the packet has passed the integrity check, while a mismatch indicates a CRC error.

The Packet Counter monitors important packet-processing events. It maintains counters such as total packets received, valid packets, CRC errors, and FIFO overflow events. These values are exposed through memory-mapped registers.

The VeeR EL2 processor acts as the system controller. It configures the IPs, reads their status and result registers, performs application-level processing of valid packet information where required, and reports the final system status through UART.

1.5 Packet Processing Concept

The packet format used for the initial implementation is conceptually:

| Packet Data | CRC |
| --- | --- |

The RTL testbench generates both the packet data and the CRC value. The CRC is calculated using the same predefined CRC algorithm that will be implemented in the CRC Checker.

For example:

Packet Data = 10110110

CRC     	= 11001010

Complete Packet = 10110110 | 11001010

The Packet FIFO stores the complete packet without modifying its contents.

The CRC Checker then processes the packet data and calculates the CRC independently:

Received Data

|

CRC Calculation

|

Calculated CRC

|

Received CRC ----------> Compare

|

+------+------+

|                   |

Match    	Mismatch

|                     |

CRC PASS   	CRC ERROR

A CRC mismatch does not recover or correct the corrupted packet. It only indicates that the received data is not consistent with its CRC. The current project therefore treats a failed packet as invalid and records the error. Retransmission or error-correcting codes may be considered as future extensions but are not part of the initial implementation.

1.6 Valid and Corrupted Packet Handling

When a packet passes the CRC check, it is considered a valid packet and can be used by the application.

Packet → FIFO → CRC Checker → CRC PASS

|

Valid Packet

|

Packet Counter

|

RISC-V

When a packet fails the CRC check, it is marked as invalid and is not treated as valid application data.

Packet → FIFO → CRC Checker → CRC ERROR

|

Invalid Packet

|

Error Counter

|

RISC-V

The RISC-V processor can read the resulting status and statistics and report them through UART.

For example:

Packet 1 : CRC PASS

Packet 2 : CRC PASS

Packet 3 : CRC ERROR

Packet 4 : CRC PASS

Total Packets : 4

Valid Packets : 3

CRC Errors	: 1

FIFO Overflow: 0

1.7 Role of Each Major Hardware Block

VeeR EL2 RISC-V Processor

The VeeR EL2 processor is the central programmable controller of the SoC. It executes the embedded application and accesses the memories and peripheral/IP registers through the AXI4 interconnect.

The processor is responsible for configuration, control, status monitoring, decision-making, and UART reporting. The VeeR EL2 processor acts as the AXI4 master of the SoC and communicates with memory-mapped memories, peripherals, and application-specific IPs through the AXI4 interconnect. The SoC interconnect is AXI4. The exact VeeR EL2 bus-wrapper/interface connection shall be adapted according to the faculty-provided VeeR EL2 RTL. The VeeR EL2 documentation identifies system-bus interfaces and a DMA slave interface as part of the core complex.

Packet FIFO

The Packet FIFO provides temporary hardware storage for incoming packet data. It decouples the packet arrival process from packet processing and provides status information such as empty, full, overflow, and underflow.

CRC Checker

The CRC Checker performs hardware-based packet integrity verification. It calculates the CRC from received packet data and compares the calculated value with the CRC supplied with the packet. The result is reported as a valid/pass or error/fail condition.

Packet Counter

The Packet Counter maintains packet-processing statistics. It can count received packets, successfully processed packets, CRC errors, and FIFO overflow events. The processor reads these counters through memory-mapped registers.

UART

UART provides the external reporting interface. The RISC-V software writes status information to the UART so that packet-processing results can be observed in the simulation console.

Timer

The Timer provides timing functionality required by the SoC and application software. It can also be used for periodic software activity or timing measurements.

GPIO

GPIO provides general-purpose digital input/output capability. It can be used for simple external control inputs and status indications during hardware demonstration or simulation.

1.8 Real-Life Applications

The underlying problem addressed by this project occurs in many systems that receive and process digital data.

Network Communication Systems

Ethernet and other packet-based communication systems continuously receive packets. Hardware buffering is required to handle differences between packet arrival and processing rates, while CRC/FCS mechanisms are used to detect transmission errors.

The proposed project models a simplified receive-side packet-processing architecture without implementing a physical Ethernet PHY, MAC, or complete network stack. This follows the reference architecture approach of injecting packet stimulus directly from a testbench rather than requiring physical networking hardware.

Routers and Network Switches

Routers and switches receive packets from multiple sources and temporarily buffer them before processing or forwarding. Hardware counters are also used to monitor packet traffic, errors, and buffer conditions.

Embedded Communication Systems

Embedded controllers communicating with other devices may receive bursts of data while the processor is executing other tasks. A hardware FIFO allows received information to wait safely until it can be processed.

FPGA-Based SoCs

In FPGA-based SoCs, dedicated RTL accelerators are commonly used for repetitive data-path operations while a processor handles configuration and higher-level control. This project demonstrates that hardware/software partition using the VeeR EL2 processor and dedicated packet-processing IPs.

IoT and Edge Devices

IoT and edge devices may receive sensor, telemetry, or communication packets that need to be buffered and checked before being passed to application software.

Industrial and Control Systems

Industrial communication systems require reliable transfer of control and monitoring data. Detecting corrupted packets before application-level processing prevents invalid data from being treated as correct system information.

1.9 Project Scope

The initial implementation includes:

• Faculty-provided VeeR EL2 RISC-V processor.

• Instruction Memory.

• Data Memory.

• AXI4 System-bus interconnect.

• UART.

• Timer.

• GPIO.

• Packet FIFO.

• CRC Checker.

• Packet Counter.

• Memory-mapped control and status registers.

• RTL testbench for packet generation.

• CRC generation within the testbench for verification.

• Valid and corrupted packet test cases.

• Embedded software running on the RISC-V processor.

• UART-based result reporting.

• RTL simulation and waveform-based verification.

The initial implementation does not require a physical Ethernet interface, Ethernet PHY, TCP/IP stack, Linux/RTOS, or DMA. DMA can be considered as a future enhancement for transferring valid packet data to memory without requiring continuous processor involvement. This is consistent with the reference architecture's treatment of DMA as an out-of-scope initial feature and a possible later performance extension.

1.10 Expected Outcome

The final SoC is expected to demonstrate a complete hardware/software packet-processing system in which:

1. The RTL testbench generates packet data and CRC.

2. The SoC accepts the incoming packet.

3. The Packet FIFO buffers the packet.

4. The CRC Checker verifies packet integrity.

5. Valid and corrupted packets are distinguished.

6. The Packet Counter records packet and error statistics.

7. The RISC-V processor accesses the hardware results through memory-mapped registers using the AXI4 interconnect.

8. The embedded application processes the obtained status.

9. UART reports the final packet-processing results.

10. RTL simulation and waveforms provide evidence that the complete system operates correctly.

The resulting architecture demonstrates hardware packet buffering, hardware data-integrity verification, hardware event monitoring, processor-controlled IP integration, memory-mapped communication, and hardware/software co-operation within a RISC-V based SoC.

Important implementation rule: all register addresses, exact bus signals, CRC parameters, packet format, interrupt wiring, and IP-level signal widths defined in later sections of this specification shall be treated as the implementation baseline. Any item dependent on the exact faculty-provided VeeR EL2 RTL shall be confirmed against that RTL before the final design is frozen. This follows the reference architecture's requirement to revalidate processor/bus-dependent details against the supplied VeeR build.

2. BLOCK DIAGRAM:

3. SIGNAL LIST:

3.1 Signal List Overview

The SoC interfaces are divided into external SoC signals and internal IP signals. External signals connect the SoC to the RTL testbench and external interfaces, while internal signals provide communication between the VeeR EL2 processor, system bus, memories, peripherals, and application-specific IPs.

The SoC uses AXI4 as the primary memory-mapped system interconnect. The exact AXI4 signal widths and configuration shall be matched to the selected VeeR EL2 interface, AXI4 interconnect configuration, memories, peripherals, and application-specific IP implementations.

3.2 External SoC Input Signals

| Signal Name | Direction | Width | Purpose |
| --- | --- | --- | --- |
| HCLK | Input | 1 | Provides the main system clock for synchronous operation of the SoC. |
| HRESETn | Input | 1 | Active-low global reset signal used to initialize the processor, memories, peripherals, and application-specific IPs. |
| PKT_DATA_IN | Input | Parameterized | Carries incoming packet data from the RTL testbench to the Packet FIFO. |
| PKT_VALID_IN | Input | 1 | Indicates that valid packet data is available on PKT_DATA_IN. |
| PKT_LAST_IN | Input | 1 | Indicates the end of the current packet. |
| GPIO_IN | Input | Parameterized | Provides external digital input signals to the GPIO peripheral. |

3.3 External SoC Output Signals

| Signal Name | Direction | Width | Purpose |
| --- | --- | --- | --- |
| PKT_READY_OUT | Output | 1 | Indicates that the SoC is ready to accept incoming packet data. |
| UART_TX | Output | 1 | Transmits packet-processing results, CRC status, packet counts, and error information through UART. |
| GPIO_OUT | Output | Parameterized | Provides general-purpose digital output signals for system status indication or external control. |
| GPIO_OUT | Output | Parameterized | Provides general-purpose digital output signals for system status indication or external control. |

3.4 Packet FIFO Interface Signals

The Packet FIFO provides temporary storage for incoming packet data. Its interface contains packet data signals, control signals, and FIFO status signals.

| Signal Name | Direction | Width | Purpose |
| --- | --- | --- | --- |
| FIFO_DATA_IN | Input | Parameterized | Receives packet data from the external packet input interface. |
| FIFO_VALID_IN | Input | 1 | Indicates that valid data is being presented to the FIFO. |
| FIFO_LAST_IN | Input | 1 | Indicates the final data element of the current packet. |
| FIFO_READY_OUT | Output | 1 | Indicates that the FIFO is ready to accept new packet data. |
| FIFO_DATA_OUT | Output | Parameterized | Provides buffered packet data to the CRC Checker. |
| FIFO_VALID_OUT | Output | 1 | Indicates that valid packet data is available at the FIFO output. |
| FIFO_LAST_OUT | Output | 1 | Indicates the final data element of the packet at the FIFO output. |
| FIFO_READ_EN | Input | 1 | Requests the FIFO to provide or remove the next stored data element. |
| FIFO_EMPTY | Output | 1 | Indicates that the FIFO contains no stored data. |
| FIFO_FULL | Output | 1 | Indicates that the FIFO cannot accept additional data. |
| FIFO_OVERFLOW | Output | 1 | Indicates that a write was attempted when the FIFO was full. |
| FIFO_UNDERFLOW | Output | 1 | Indicates that a read was attempted when the FIFO was empty. |
| FIFO_LEVEL | Output | Parameterized | Indicates the current number of stored FIFO entries. |

3.5 CRC Checker Interface Signals

| Signal Name | Direction | Width | Purpose |
| --- | --- | --- | --- |
| CRC_DATA_IN | Input | Parameterized | Receives packet data from the Packet FIFO. |
| CRC_VALID_IN | Input | 1 | Indicates that valid packet data is available for CRC processing. |
| CRC_LAST_IN | Input | 1 | Indicates the end of the packet and triggers completion of the CRC verification operation. |
| CRC_VALUE_IN | Input | Parameterized | Carries the CRC value received with the packet for comparison. |
| CRC_START | Input | 1 | Starts or enables CRC processing for a packet. |
| CRC_RESULT | Output | Parameterized | Provides the calculated CRC value. |
| CRC_PASS | Output | 1 | Indicates that the calculated CRC matches the received CRC. |
| CRC_ERROR | Output | 1 | Indicates that the calculated CRC does not match the received CRC. |
| CRC_DONE | Output | 1 | Indicates that CRC verification for the current packet has completed. |

The CRC Checker receives packet data from the Packet FIFO and verifies the integrity of the received packet.

3.6 Packet Counter Interface Signals

The Packet Counter records packet-processing and error statistics.

| Signal Name | Direction | Width | Purpose |
| --- | --- | --- | --- |
| PKT_COUNT_EN | Input | 1 | Enables packet-counting activity. |
| PKT_VALID_EVENT | Input | 1 | Indicates that a packet has successfully passed CRC verification. |
| CRC_ERROR_EVENT | Input | 1 | Indicates that a packet has failed CRC verification. |
| FIFO_OVERFLOW_EVENT | Input | 1 | Indicates that a FIFO overflow has occurred. |
| FIFO_UNDERFLOW_EVENT | Input | 1 | Indicates that a FIFO underflow has occurred. |
| TOTAL_PKT_COUNT | Output | Parameterized | Provides the total number of received or processed packets. |
| VALID_PKT_COUNT | Output | Parameterized | Provides the number of packets that passed CRC verification. |
| CRC_ERROR_COUNT | Output | Parameterized | Provides the number of packets that failed CRC verification. |
| FIFO_ERROR_COUNT | Output | Parameterized | Provides the number of recorded FIFO error events. |

3.7 AXI4 System Interconnect Interface Signals

| Channel | Signal Name | Direction (From Master View) | Width | Purpose / Description |
| --- | --- | --- | --- | --- |
| Write Address Channel (AW) | AWID | Output | ID_WIDTH | Write address ID tag. Used to identify write transactions. |
| Write Address Channel (AW) | AWADDR | Output | ADDR_WIDTH | Write address. Specifies the address for the write transaction. |
| Write Address Channel (AW) | AWLEN | Output | 8 | Write burst length. Number of transfers in the burst (0 = 1 transfer, 1 = 2 transfers, ..., 255 = 256 transfers). |
| Write Address Channel (AW) | AWSIZE | Output | 3 | Write transfer size. Indicates the size of each transfer in the burst (2^AWSIZE bytes). |
| Write Address Channel (AW) | AWBURST | Output | 2 | Write burst type. 00 = FIXED, 01 = INCR, 10 = WRAP. |
| Write Address Channel (AW) | AWLOCK | Output | 2 | Write lock type. 00 = Normal, 01 = Exclusive, others = Reserved. |
| Write Address Channel (AW) | AWCACHE | Output | 4 | Write cache type. Defines cache attributes for the transaction. |
| Write Address Channel (AW) | AWPROT | Output | 3 | Write protection type. [2] = Privileged, [1] = Secure, [0] = Data/Instruction. |
| Write Address Channel (AW) | AWQOS | Output | 4 | Write quality of service. |
| Write Address Channel (AW) | AWREGION | Output | 4 | Write region identifier. |
| Write Address Channel (AW) | AWVALID | Output | 1 | Write address valid. Indicates that the address and control information are valid. |
| Write Address Channel (AW) | AWREADY | Input | 1 | Write address ready. Indicates that the slave is ready to accept the write address. |
| Write Data Channel (W) | WDATA | Output | DATA_WIDTH | Write data. Carries the write data. |
| Write Data Channel (W) | WSTRB | Output | DATA_WIDTH/8 | Write strobes. Indicates which byte lanes of WDATA are valid. |
| Write Data Channel (W) | WLAST | Output | 1 | Write last. Indicates the last data transfer in the burst. |
| Write Data Channel (W) | WVALID | Output | 1 | Write data valid. Indicates that write data and control are valid. |
| Write Data Channel (W) | WREADY | Input | 1 | Write data ready. Indicates that the slave is ready to accept write data. |
| Write Response Channel (B) | BID | Input | ID_WIDTH | Write response ID tag. Returns the ID of the write transaction. |
| Write Response Channel (B) | BRESP | Input | 2 | Write response. 00 = OKAY, 01 = EXOKAY, 10 = SLVERR, 11 = DECERR. |
| Write Response Channel (B) | BVALID | Input | 1 | Write response valid. Indicates that the response is valid. |
| Write Response Channel (B) | BREADY | Output | 1 | Write response ready. Indicates that the master is ready to accept the response. |
| Read Address Channel (AR) | ARID | Output | ID_WIDTH | Read address ID tag. Used to identify read transactions. |
| Read Address Channel (AR) | ARADDR | Output | ADDR_WIDTH | Read address. Specifies the address for the read transaction. |
| Read Address Channel (AR) | ARLEN | Output | 8 | Read burst length. Number of transfers in the burst. |
| Read Address Channel (AR) | ARSIZE | Output | 3 | Read transfer size. Indicates the size of each transfer. |
| Read Address Channel (AR) | ARBURST | Output | 2 | Read burst type. 00 = FIXED, 01 = INCR, 10 = WRAP. |
| Read Address Channel (AR) | ARLOCK | Output | 2 | Read lock type. 00 = Normal, 01 = Exclusive, others = Reserved. |
| Read Address Channel (AR) | ARCACHE | Output | 4 | Read cache type. Defines cache attributes for the transaction. |
| Read Address Channel (AR) | ARPROT | Output | 3 | Read protection type. [2] = Privileged, [1] = Secure, [0] = Data/Instruction. |
| Read Address Channel (AR) | ARQOS | Output | 4 | Read quality of service. |
| Read Address Channel (AR) | ARREGION | Output | 4 | Read region identifier. |
| Read Address Channel (AR) | ARVALID | Output | 1 | Read address valid. Indicates that the address and control information are valid. |
| Read Address Channel (AR) | ARREADY | Input | 1 | Read address ready. Indicates that the slave is ready to accept the read address. |
| Read Data Channel (R) | RID | Input | ID_WIDTH | Read data ID tag. Returns the ID of the read transaction. |
| Read Data Channel (R) | RDATA | Input | DATA_WIDTH | Read data. Carries the read data. |
| Read Data Channel (R) | RRESP | Input | 2 | Read response. 00 = OKAY, 01 = EXOKAY, 10 = SLVERR, 11 = DECERR. |
| Read Data Channel (R) | RLAST | Input | 1 | Read last. Indicates the last data transfer in the burst. |
| Read Data Channel (R) | RVALID | Input | 1 | Read data valid. Indicates that read data and response are valid. |
| Read Data Channel (R) | RREADY | Output | 1 | Read data ready. Indicates that the master is ready to accept the read data. |
| Reset and Clock (Optional but recommended) | ACLK | Output | 1 | AXI clock. Used to sample all AXI signals. |
| Reset and Clock (Optional but recommended) | ARESETn | Output | 1 | Active-low reset. Resets all AXI interface logic. |

Notes:

• ID_WIDTH, ADDR_WIDTH, and DATA_WIDTH are system parameters defined in the SoC.

• All VALID signals are asserted by the source and must remain stable until READY is asserted.

• All READY signals are driven by the destination and indicate acceptance capability.

• AXI4 supports multiple outstanding transactions using ID tags.

The AXI4 interconnect provides the primary memory-mapped communication path between the VeeR EL2 processor and the memories, peripherals, and application-specific IPs in the SoC.

The VeeR EL2 processor acts as the AXI4 master and initiates memory-mapped read and write transactions. The AXI4 interconnect performs address decoding and routes each transaction to the selected memory, peripheral, or application-specific IP.

The AXI4 interconnect supports the AXI4 write-address, write-data, write-response, read-address, and read-data channels using VALID/READY handshaking.

The SoC uses an AXI4 memory-mapped interconnect for communication between the VeeR EL2 processor and the memory, peripheral, and application-specific IP blocks. The selected AXI4 interconnect supports AXI write-address, write-data, write-response, read-address, and read-data channels with VALID/READY handshaking.

The following signals represent the logical functions required by the SoC interconnect. RTL. The following signals represent the logical functions required by the SoC interconnect.

The exact signal names, widths, transaction timing, response encoding, and protocol behavior shall not be assumed from these logical names. They shall be derived from the actual VeeR EL2 bus interface used in the final implementation.

The selected interconnect implements the full AXI4 memory-mapped protocol. AXI4-Lite is not used as the primary SoC interconnect.

3.7.1 AXI4 Interconnect Configuration

The SoC uses AXI4 as the primary memory-mapped system interconnect. The VeeR EL2 RISC-V processor acts as the upstream AXI4 master and initiates read and write transactions to access the memories, peripherals, and application-specific IPs.

The selected axi_interconnect RTL provides address decoding, transaction routing, arbitration, and response forwarding between the processor-side AXI4 interface and the connected target interfaces.

For the initial SoC architecture, the AXI4 interconnect shall be configured with one upstream processor interface and eight downstream target interfaces.

The AXI4 target interfaces are assigned as follows:

M00 – Instruction Memory

M01 – Data Memory

M02 – UART

M03 – Timer

M04 – GPIO

M05 – Packet FIFO

M06 – CRC Checker

M07 – Packet Counter

The AXI4 interconnect is used primarily for processor control, configuration, memory access, and status/result retrieval. The primary packet data path remains independent of the AXI4 control path and follows:

RTL Packet Source → Packet FIFO → CRC Checker → Packet Counter.

The exact AXI4 signal widths and processor-side interface adaptation shall be matched to the faculty-provided VeeR EL2 RTL and the final integrated IP implementations.

3.8 Memory Interface Signals

Instruction Memory and Data Memory provide storage for the RISC-V application program and application data.

Instruction Memory

| Signal Name | Direction | Width | Purpose |
| --- | --- | --- | --- |
| IMEM_ADDR | Input | Parameterized | Specifies the instruction memory address accessed by the processor. |
| IMEM_RDATA | Output | Parameterized | Returns the instruction stored at the requested address. |
| IMEM_EN | Input | 1 | Enables an instruction memory access. |

Data Memory

| Signal Name | Direction | Width | Purpose |
| --- | --- | --- | --- |
| DMEM_ADDR | Input | Parameterized | Specifies the data memory address accessed by the processor. |
| DMEM_WDATA | Input | Parameterized | Carries data to be written into data memory. |
| DMEM_RDATA | Output | Parameterized | Returns data read from data memory. |
| DMEM_WRITE_EN | Input | 1 | Enables a data-memory write operation. |
| DMEM_READ_EN | Input | 1 | Enables a data-memory read operation. |
| DMEM_BYTE_EN | Input | Parameterized | Selects the valid byte lanes during a data-memory access. |

3.9 UART Interface Signals

| Signal Name | Direction | Width | Purpose |
| --- | --- | --- | --- |
| UART_TX | Output | 1 | Serial transmit signal used to report packet-processing results. |
| UART_RX | Input | 1 | Serial receive signal for receiving optional control/configuration information. |
| UART_TX_READY | Output | 1 | Indicates that the UART transmitter is ready to accept new data. |
| UART_TX_VALID | Input | 1 | Indicates that valid data is being supplied to the UART transmitter. |
| UART_TX_DATA | Input | Parameterized | Carries the data to be transmitted through UART. |

3.10 GPIO Interface Signals

| Signal Name | Direction | Width | Purpose |
| --- | --- | --- | --- |
| GPIO_IN | Input | Parameterized | Receives external digital input values. |
| GPIO_OUT | Output | Parameterized | Drives external digital output values. |
| GPIO_OE | Output | Parameterized | Controls the direction or output enable of GPIO pins where supported. |

GPIO is not part of the primary packet-processing data path. It is included as a general-purpose SoC peripheral for external control, status indication, and hardware demonstration.

3.11 Clock and Reset Signals

The SoC uses a common system clock and reset mechanism for synchronous operation.

All synchronous state elements shall operate according to the selected clocking scheme. Reset behavior shall be consistent across the processor, interconnect, memories, peripherals, FIFO, CRC Checker, and Packet Counter.

3.12 Signal Interaction Summary

The primary packet data path is:

PKT_DATA_IN → Packet FIFO → CRC Checker → Packet Counter

The processor control and monitoring path is:

VeeR EL2 → AXI4 Interconnect → IP Control/Status Registers

and:

IP Status/Results → AXI4 Interconnect → VeeR EL2

The reporting path is:

VeeR EL2 → UART → UART_TX

The external GPIO path is:

GPIO_IN → GPIO Peripheral → VeeR EL2

and:

VeeR EL2 → GPIO Peripheral → GPIO_OUT

The Packet FIFO status signals such as FIFO_EMPTY, FIFO_FULL, FIFO_OVERFLOW, and FIFO_UNDERFLOW are used internally for buffering control and error monitoring. The CRC result signals CRC_PASS, CRC_ERROR, and CRC_DONE provide the integrity-check result to the monitoring and control logic.

3.13 Signal Naming and Implementation Rules

1. All signal names shall use a consistent naming convention throughout the RTL.

2. Signal direction shall be defined relative to the module or IP being described.

3. Signal widths shall be parameterized where the final width depends on the selected VeeR EL2 configuration or IP implementation.

4. The VeeR EL2 processor shall be connected to the selected AXI4 interconnect using the actual interface signals provided by the faculty-supplied VeeR EL2 RTL. Any required processor-side AXI4 adaptation or wrapper shall be implemented according to the actual VeeR RTL.

5. The Packet FIFO shall expose sufficient status information to detect full, empty, overflow, and underflow conditions.

6. The CRC Checker shall provide an explicit indication of CRC completion and CRC pass/error status.

7. Packet Counter events shall be generated only from well-defined packet-processing events.

8. External packet input signals shall be driven by the RTL testbench during simulation.

9. GPIO signals shall remain independent of the primary packet data path.

10. Any signal not required by the final selected IP implementation shall be removed from the final RTL interface rather than being left as an unused interface.

11. The final signal widths, timing relationships, reset polarity, and handshake behavior shall be verified against the integrated RTL before implementation and verification are finalized.

4. CONTROL PATH

4.1 Control Path Overview

The control path is responsible for controlling and coordinating the operation of the RISC-V processor, system bus, memories, peripherals, and application-specific IPs. The VeeR EL2 RISC-V processor executes the embedded application and communicates with the memory-mapped IPs through the AXI4 interconnect. The processor configures the required IPs, reads their status and result registers, monitors packet-processing events, and controls the overall application flow.

The control path is separate from the primary packet data path. Packet data moves through the Packet FIFO and CRC Checker, while the control path is mainly used for configuration, status monitoring, register access, and decision-making.

The overall control flow is:

VeeR EL2 RISC-V → AXI4 interconnect → Address Decoder/Interconnect → Selected IP Register Interface

and for status monitoring:

Selected IP Status/Result → System Bus → VeeR EL2 RISC-V

4.2 Processor as the Main Controller

The VeeR EL2 RISC-V processor acts as the main programmable controller of the SoC. After reset, the processor starts executing the embedded application from Instruction Memory. The software initializes the required peripherals and application-specific IPs before packet processing begins.

The processor is responsible for:

1. Initializing the SoC peripherals and application-specific IPs.

2. Configuring the Packet FIFO, CRC Checker, and Packet Counter where configuration is required.

3. Reading FIFO status information.

4. Monitoring CRC verification results.

5. Reading packet and error counters.

6. Handling error conditions.

7. Processing valid packet information at the software level where required.

8. Reporting packet-processing results through UART.

9. Providing optional GPIO-based status indication.

The processor does not perform the repetitive CRC calculation itself. The CRC calculation and packet buffering are performed by dedicated hardware IPs.

4.3 AXI4 Interconnect Control

The system bus provides the control and register-access path between the VeeR EL2 processor and the memory-mapped components.

When the processor executes a load or store instruction targeting a memory-mapped address, the corresponding bus transaction is generated. The system interconnect decodes the address and selects the appropriate memory or peripheral.

The logical operation is:

Processor generates address

|

System Bus Transaction

|

Address Decode

|

+-------------------+

|                        |

Memory      Peripheral/IP

|               	     |

+---------+--------+

|

Response Data

|

RISC-V

The AXI4 interconnect provides the memory-mapped control and status path between the VeeR EL2 processor and the connected memories, peripherals, and application-specific IPs.

For a write transaction, the VeeR EL2 AXI4 master provides the write address and write data through the AXI4 write-address and write-data channels. The AXI4 interconnect decodes the address and routes the transaction to the selected slave. The selected slave returns a write response through the AXI4 write-response channel.

For a read transaction, the VeeR EL2 AXI4 master provides the read address through the AXI4 read-address channel. The AXI4 interconnect decodes the address and routes the request to the selected slave. The selected slave returns the requested data through the AXI4 read-data channel.

AXI4 VALID and READY signals are used to control transfer acceptance between the master, interconnect, and selected slave.

4.4 Address Decoding and IP Selection

Each memory, peripheral, and application-specific IP is assigned a unique memory-mapped address range.

When the RISC-V processor accesses an address, the AXI4 interconnect determines which component owns that address.

The logical control flow is:

Processor Address

|

AXI4 Address Transaction

↓

AXI4 Interconnect

↓

Address Decode

↓

Selected Target

|

+---- Instruction Memory

|

+---- Data Memory

|

+---- UART

|

+---- Timer

|

+---- GPIO

|

+---- Packet FIFO

|

+---- CRC Checker

|

+---- Packet Counter

Only the selected component responds to the transaction. The other components remain inactive for that transaction.

The final address ranges shall match the memory map defined for the integrated SoC and shall be treated consistently by both RTL and RISC-V software.

4.5 Packet FIFO Control

The Packet FIFO is primarily controlled by packet input handshaking and FIFO status conditions. The external testbench provides packet data and valid information. The FIFO accepts data when it is ready and has available storage.

The FIFO control operation is:

Packet Data + Valid

|

FIFO Ready?

/ 	       \

Yes  	       No

|            	|

Store Data          Wait

|

Update FIFO Status

The FIFO generates status information such as FIFO_EMPTY, FIFO_FULL, FIFO_OVERFLOW, FIFO_UNDERFLOW, and FIFO_LEVEL.

The control logic uses these conditions to prevent invalid read or write operations. An attempted write when the FIFO is full is treated as an overflow event. An attempted read when the FIFO is empty is treated as an underflow event.

The processor can read the FIFO status through its memory-mapped registers.

4.6 CRC Checker Control

The CRC Checker is controlled by the packet-processing sequence. When valid packet data becomes available from the FIFO, the CRC Checker processes the packet data and calculates the CRC.

The control sequence is:

FIFO provides valid data

|

CRC Checker starts processing

|

Process packet data

|

End of packet detected

|

CRC calculation completed

|

Compare calculated CRC with received CRC

|

+-------------------------------+

|                  			|

CRC PASS           	CRC ERROR

The CRC Checker generates status signals including CRC_PASS, CRC_ERROR, CRC_DONE, and the calculated CRC result.

The processor can read the CRC status and result through the corresponding memory-mapped registers.

4.7 Packet Counter Control

The Packet Counter receives event signals from the packet-processing hardware. It updates the corresponding counters when defined events occur.

The main events include:

1. Packet received or processed.

2. Packet successfully passing CRC verification.

3. Packet failing CRC verification.

4. FIFO overflow.

5. FIFO underflow, if included in the selected implementation.

The logical control operation is:

Packet/Event

|

Event Detection

|

+---------------+-----------------------+

|      	           |                  	|

Total Count      Valid Count      Error Count

|

Memory-Mapped Registers

|

RISC-V Reads Statistics

The counters provide the processor with a hardware-generated summary of packet-processing activity.

4.8 Processor Control of Packet Processing

The RISC-V software coordinates the overall packet-processing operation.

The initial control sequence is:

1. Reset the SoC.

2. Initialize the required peripherals.

3. Initialize the Packet FIFO.

4. Initialize the CRC Checker.

5. Initialize or clear the Packet Counter.

6. Enable packet processing.

7. Wait for packet-processing activity or monitor status.

8. Read CRC verification results.

9. Read packet and error statistics.

10. Process valid packet information if required.

11. Report the results through UART.

12. Continue monitoring for additional packets.

The processor therefore acts as the supervisor of the hardware packet-processing system rather than directly handling every packet-data operation.

4.9 Valid Packet Control Flow

When the CRC Checker determines that a packet is valid, the control logic generates a CRC_PASS event.

The control sequence is:

CRC_PASS

|

Packet marked valid

|

Packet Counter updates valid packet count

|

RISC-V reads status/result

|

Valid packet information can be processed or stored

|

UART reports the result if required

The valid packet data is not modified by the CRC checking operation. The CRC Checker only determines whether the received packet data is consistent with the received CRC.

4.10 Corrupted Packet Control Flow

When the calculated CRC does not match the received CRC, the CRC Checker generates a CRC_ERROR event.

The control sequence is:

CRC_ERROR

|

Packet marked invalid

|

Packet Counter increments CRC error count

|

RISC-V reads error status

|

Invalid packet is discarded or excluded from valid application processing

|

UART reports the CRC error if required

The CRC Checker does not attempt to reconstruct or correct the corrupted packet. CRC provides error detection rather than error correction.

4.11 FIFO Error Control

The control path also monitors FIFO error conditions.

FIFO overflow occurs when an attempt is made to write packet data while the FIFO is full.

FIFO underflow occurs when an attempt is made to read packet data while the FIFO is empty.

The logical error flow is:

FIFO Error

|

Error Event Generated

|

Packet Counter Updates Error Count

|

RISC-V Reads Error Status

|

Application Reports or Handles Error

The exact error response shall be defined by the final RTL implementation. At minimum, the error condition shall be observable through the appropriate status or counter register.

4.12 UART Control

The UART is controlled by the RISC-V software. After obtaining packet-processing results, the processor converts the required information into a printable format and sends it through the UART transmit interface.

The logical flow is:

RISC-V

|

Prepare Result Message

|

UART Data Register

|

UART Transmitter

|

UART_TX

|

vSimulation Console

Example information reported through UART may include:

Total Packets

Valid Packets

CRC Errors

FIFO Overflow Events

FIFO Underflow Events

Current Packet Status

4.13 GPIO Control

GPIO is controlled through its memory-mapped registers. The RISC-V software may read GPIO inputs and write GPIO outputs.

GPIO is not part of the primary packet data path. It is used for general-purpose external control and status indication.

The control flow is:

GPIO_IN → GPIO Peripheral → RISC-V

and:

RISC-V → GPIO Peripheral → GPIO_OUT

GPIO outputs may optionally be used to indicate simple system states such as CRC PASS, CRC ERROR, or system activity.

4.14 Timer Control

The Timer is controlled through its memory-mapped registers. The RISC-V software may configure the timer and use its timing information for periodic operations or application-level timing functions.

The Timer is not directly involved in the primary packet data path.

4.15 Reset Control

The HRESETn signal places the SoC into its initial state.

During reset:

1. The RISC-V processor is placed in its reset state.

2. FIFO state and pointers are initialized.

3. CRC Checker state is initialized.

4. Packet Counter values are initialized according to the selected reset behavior.

5. UART state is initialized.

6. Timer state is initialized.

7. GPIO state is initialized.

8. Memory and interconnect control logic are placed into their defined reset states.

After reset is released, the processor begins executing the application and initializes the system.

4.16 Overall Control Path

The complete control path can be summarized as:

HRESETn

|

VeeR EL2 Initialization

|

AXI4 Interconnect

|

Peripheral and IP Configuration

|

Enable Packet Processing

|

Monitor FIFO / CRC / Counter Status

|

|

|

|

|

Read Status and Results

|

+-----------------------------+

|                      	          |

Valid Packet          	Error Condition

|                      	          |

Process/Store         Record Error

|                      	          |

+-------------+--------------+

|

RISC-V

|

UART

The control path therefore provides the supervisory mechanism that connects the processor software with the hardware packet-processing functions.

4.17 Control Path Design Requirements

1. The VeeR EL2 processor shall be the main programmable controller of the SoC.

2. All memory-mapped IPs shall have uniquely defined address ranges.

3. The AXI4 Interconnect shall route processor transactions to the correct target.

4. Control registers shall be writable by the processor only where configuration is required.

5. Status and counter registers shall be readable by the processor.

6. FIFO status shall prevent or identify invalid read and write operations.

7. CRC status shall clearly distinguish CRC_PASS, CRC_ERROR, and CRC_DONE conditions.

8. Packet Counter updates shall be based on clearly defined hardware events.

9. Reset behavior shall be deterministic for all control and status registers.

10. UART reporting shall be performed by RISC-V software.

11. GPIO and Timer shall remain independent of the primary packet-processing data path unless explicitly required by the application.

12. AXI4 is the selected primary SoC interconnect. The exact AXI4 signal widths, register addresses, timing, and processor-side interface adaptation shall be finalized according to the integrated VeeR EL2 RTL and selected AXI4 interconnect configuration.